EX NO: 7

CALCULATION AND VERIFICATION OF FREQUENCY DEVIATION AND MODULATION INDEX OF FM SIGNALS USING MATLAB SIMULATION

Objectives:

1. To generate a Frequency Modulated (FM) signal in MATLAB using specified carrier and message signal parameters.

2. To calculate and verify the frequency deviation of the FM signal from the given carrier frequency and modulating signal parameters.

3. To determine and verify the modulation index of the FM signal using the relationship between frequency deviation and message frequency.

4. To analyze the effect of message frequency on frequency deviation and modulation index, and verify their relationship through MATLAB simulation

Procedure:

1. Open MATLAB and create a new script file.

2. Clear the command window, workspace variables, and close all previously opened figure windows using: Clc; Clear; close all;

3. Define the message signal parameters:

● Message amplitude A_m=2V

● Message frequency f_m=500Hz 4. Define the carrier signal parameters:

● Carrier amplitude A_c=1V

● Carrier frequency f_c=10000Hz

5. Define the frequency sensitivity constant k_f, sampling frequency f_s, and simulation duration T

6. Generate the time vector using: t = 0:1/fs:T;

7. Generate the message signal using: m = Am*cos(2*pi*fm*t); and observe the sinusoidal waveform.

8. Calculate the frequency deviation of the FM signal using: Δf=kf. Am

9. Calculate the modulation index using: β=Δf/fm

10. Generate the FM signal using: fm_signal = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t)); where the carrier frequency varies according to the message signal.

11. Plot the message signal in the first subplot and label the axes appropriately.

12. Plot the FM signal in the second subplot and observe the variation in instantaneous frequency.

13. Display the carrier frequency, message frequency, message amplitude, and frequency sensitivity in the MATLAB command window using fprintf statements.

14. Display the calculated frequency deviation and modulation index values in the command window.

15. Verify the modulation index by recomputing it using: β=Δf/fm and compare the result with the previously calculated value.

16. Use a conditional statement (if-else) to check whether both modulation index values are equal within a small tolerance.

17. Display the verification result as:

● "Verification Successful" if both values match.

● "Verification Failed" otherwise.

18. Execute the MATLAB program and observe the message signal, FM signal, calculated frequency deviation, modulation index, and verification result.

PROGRAM :

% Calculation and Verification of Frequency Deviation

% and Modulation Index of FM Signal

clc;

clear;

close all;

%% Message Signal Parameters

Am = 2; % Message amplitude (V)

fm = 500; % Message frequency (Hz)

%% Carrier Signal Parameters

Ac = 1; % Carrier amplitude (V)

fc = 10000; % Carrier frequency (Hz)

%% FM Parameters

kf = 250; % Frequency sensitivity (Hz/V)

fs = 100000; % Sampling frequency (Hz)

T = 0.01; % Simulation duration (s)

%% Time Vector

t = 0:1/fs:T;

%% Message Signal

m = Am*cos(2*pi*fm*t);

%% Frequency Deviation

delta_f = kf*Am;

%% Modulation Index

beta = delta_f/fm;

%% FM Signal

fm_signal = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

%% Display Parameters

fprintf('Carrier Frequency (fc) = %.0f Hz\n',fc);

fprintf('Message Frequency (fm) = %.0f Hz\n',fm);

fprintf('Message Amplitude (Am) = %.0f V\n',Am);

fprintf('Frequency Sensitivity (kf) = %.0f Hz/V\n',kf);

fprintf('\nFrequency Deviation (Delta f) = %.2f Hz\n',delta_f);

fprintf('Modulation Index (Beta) = %.2f\n',beta);

%% Verification

beta_verify = delta_f/fm;

fprintf('Verified Modulation Index = %.2f\n',beta_verify);

%% Verification  Check

tolerance = 1e-6;

if abs(beta - beta_verify) < tolerance

fprintf('\nVerification Successful\n');

else

fprintf('\nVerification Failed\n');

end

%% Plotting

figure;

subplot(2,1,1);

plot(t,m,'LineWidth',1.5);

grid on;

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude (V)');

subplot(2,1,2);

plot(t,fm_signal,'LineWidth',1);

grid on;

title('FM Signal');

xlabel('Time (s)');

ylabel('Amplitude (V)');

Output:

Result:

1. Generate and visualize carrier, message, and FM signals using MATLAB.

2. Calculate the theoretical frequency deviation and verify it from the simulated FM waveform.

3. Calculate the FM modulation index and verify the relationship β = Δf/fm.

4. Interpret the effect of changing message frequency on modulation index and verify that, for constant frequency sensitivity and message amplitude, the peak frequency deviation remains constant.